TECHNICAL METHODOLOGY AND IMPLEMENTATION REPORT: BCI-INTEGRATED UR5e ROBOTIC CONTROL IN GAZEBO

Abstract

Brain–Computer Interface (BCI) systems establish a direct communication pathway between neural activity and computational systems without requiring muscular interaction [4]. This report presents a comprehensive simulation-based methodology for implementing a Motor Imagery (MI)-driven BCI integrated with a Universal Robots UR5e robotic manipulator within the Gazebo simulation environment.

The proposed framework focuses entirely on software-level simulation, EEG signal processing, machine learning-based classification, robotic trajectory generation, and ROS 2 middleware communication. EEG signals obtained from publicly available datasets are processed through a multi-stage computational pipeline involving preprocessing, feature extraction, feature selection, classification, probabilistic decision logic, and robotic command mapping.

The implementation utilizes Python, ROS 2, MoveIt 2, Gazebo, NumPy, Scikit-learn, and BioSig to establish a modular neuro-robotic simulation architecture. Classified Motor Imagery commands are translated into robotic joint trajectories executed within a high-fidelity Gazebo simulation environment.

The project emphasizes simulation-first validation, collision-aware motion planning, confidence-threshold-based command execution, and realistic robotic movement emulation. The proposed methodology aims to achieve robust classification performance, low-latency neuro-robotic interaction, and scalable software architecture suitable for academic research in neuro-robotics and intelligent human–machine systems.

1. Introduction

Brain–Computer Interfaces (BCIs) are systems that enable communication between neural activity and external computational systems by interpreting electrophysiological brain signals [4]. Electroencephalography (EEG)-based BCIs are widely used because they are non-invasive, computationally accessible, and suitable for real-time signal acquisition [12].

Among various BCI paradigms, Motor Imagery (MI)-based systems are extensively researched due to their ability to decode imagined movement intentions without requiring actual muscular execution [1][19]. During Motor Imagery tasks, oscillatory activity in the sensorimotor cortex undergoes measurable modulation within the alpha and beta frequency bands [15]. These neurophysiological changes manifest as Event-Related Desynchronization (ERD) and Event-Related Synchronization (ERS), forming the basis for machine learning-based EEG classification.

Recent advancements in neuro-robotics have demonstrated the feasibility of integrating MI-based BCIs with robotic manipulators for intelligent robotic interaction, assistive systems, and rehabilitation-oriented applications [13]. However, direct deployment on physical robotic hardware introduces safety risks, cost constraints, and reproducibility challenges.

To address these limitations, this work adopts a simulation-first methodology utilizing the UR5e robotic manipulator model integrated with ROS 2, MoveIt 2, and Gazebo. The project focuses exclusively on software-based simulation and aims to develop a robust neuro-robotic architecture capable of converting EEG-derived mental commands into realistic robotic movements within a high-fidelity simulation environment [5].

2. System Architecture Overview

The proposed framework consists of five major computational layers.

TABLE 1: SYSTEM ARCHITECTURE OVERVIEW

| Layer | Function |
| --- | --- |
| EEG Acquisition Layer | Reads EEG datasets in GDF format |
| Signal Processing Layer | Filters and preprocesses EEG signals |
| Feature Extraction Layer | Computes discriminative EEG features |
| Classification Layer | Predicts Motor Imagery classes |
| Robotic Control | Layer Generates UR5e robotic trajectories |

The overall system workflow is represented as:

EEG → Filtering → ICA → CSP → PSD → MIBIF → SVM → ROS2 → MoveIt2 → Gazebo

The modular ROS 2 architecture enables distributed communication between EEG-processing nodes and robotic control nodes [10].

3. Main Modelling Approach: Motor Imagery Paradigm

3.1 Motor Imagery Fundamentals

Motor Imagery refers to the mental rehearsal of movement without actual physical execution [18]. During imagined movement, neural oscillations within the sensorimotor cortex exhibit measurable changes detectable using EEG recordings.

The proposed framework utilizes MI-based EEG decoding because it enables direct neuro-control without requiring muscular interaction [1].

TABLE 2: EEG FREQUENCY BANDS

| Frequency Band Range | Functional Role |
| --- | --- |
| Alpha (μ-rhythm) 8–13 Hz | Sensorimotor activity |
| Beta Rhythm 13–30 Hz | Motor planning |

[15]

3.2 ERD/ERS Neurophysiological Modulation

Motor Imagery induces characteristic oscillatory changes in cortical activity:

• Event-Related Desynchronization (ERD): Reduction in rhythmic power during motor intention [15].

• Event-Related Synchronization (ERS): Rebound increase in rhythmic activity following task completion [15].

TABLE 3: ERD/ERS RESPONSES

| Imagined Action | EEG Response |
| --- | --- |
| Left-hand imagery | ERD over right motor cortex |
| Right-hand imagery | ERD over left motor cortex |
| Foot imagery | Midline desynchronization |

These EEG modulations form the neurophysiological foundation for feature extraction and classification.

3.3 Justification for Motor Imagery Selection

Motor Imagery was selected because:

- It is non-invasive and computationally feasible [12].

- It enables direct robotic control through neural intention [13].

- It is extensively validated in BCI literature [22].

- It integrates effectively with CSP and SVM pipelines [8].

- It is highly suitable for simulation-based neuro-robotic experimentation.

4. Justification of the UR5e Robotic Manipulator Model

4.1 Selection Rationale

The UR5e robotic manipulator model was selected exclusively for simulation purposes due to its:

• Accurate URDF representation

• Native ROS 2 integration

• MoveIt 2 compatibility

• Stable kinematic structure

• Gazebo plugin support

[10]

4.2 Six-Degree-of-Freedom Kinematics

The UR5e simulation model provides six rotational joints enabling:

• Cartesian positioning

• End-effector orientation control

• Joint trajectory planning

• Multi-axis robotic movement simulation

The 6-DOF structure is suitable for emulating neuro-controlled robotic actions such as:

• Directional movement

• Simulated grasping

• End-effector positioning

• Target approach trajectories

[17]

4.3 Simulation-Centric Advantages

The UR5e simulation environment enables:

• Collision-aware testing

• Repeatable experiments

• Software-only validation

• Joint-state visualization

• Safe evaluation of misclassified commands

without requiring physical robotic deployment.

5. Justification of Gazebo Simulator

5.1 Selection of Gazebo

Gazebo was selected due to its strong compatibility with ROS 2 and support for realistic robotic simulation workflows [17].

5.2 Gazebo vs PyBullet

TABLE 4: GAZEBO VS PYBULLET

| Feature | Gazebo | PyBullet |  |
| --- | --- | --- | --- |
| Native ROS 2 Support | Excellent | Limited |  |
| MoveIt 2 Integration | Native |  | Partial |
| URDF Compatibility | Extensive | Moderate |  |
| Joint-State Simulation | High Fidelity | Moderate |  |
| Industrial Robotics Ecosystem | Strong | Limited |  |

5.3 Advantages of Gazebo

Gazebo provides:

• Rigid-body dynamics simulation

• Collision-aware environments

• Joint trajectory execution

• Sensor simulation plugins

• Real-time ROS 2 communication

These features are essential for validating neuro-robotic control architectures prior to hardware deployment [5].

6. Software Implementation Stack

6.1 Programming Language

Python was selected as the primary implementation language because of its:

• Scientific computing ecosystem

• Machine learning support

• ROS 2 integration

• Rapid prototyping capability

Approximately 94.6% of the implementation is Python-based.

6.2 Core Technologies

TABLE 5: CORE TECHNOLOGIES

| Technology | Purpose |
| --- | --- |
| Python | EEG processing and control logic |
| ROS 2 Jazzy/Humble | Middleware communication |
| Gazebo | Robotic simulation |
| MoveIt 2 | Motion planning |
| NumPy | Numerical computation |
| Scikit-learn | Machine learning |
| BioSig | EEG data handling |

6.3 ROS 2 Node Architecture

TABLE 6: ROS 2 NODE ARCHITECTURE

| ROS 2 Node | Function |
| --- | --- |
| EEG Reader Node | Reads GDF EEG datasets |
| Filtering Node | Executes preprocessing |
| Feature Extraction Node | Computes CSP and PSD features |
| Classification Node | Performs SVM classification |
| Command Mapping Node | Maps MI classes to robot commands |
| Motion Control Node | Sends trajectories to Gazebo |

7. Methodology and Algorithmic Implementation

The proposed neuro-robotic framework follows a structured signal-processing and robotic-control pipeline specifically designed for simulation-based BCI experimentation.

7.1 Signal Preprocessing Algorithms

Raw EEG signals contain noise, baseline drift, ocular artifacts, muscular interference, and environmental contamination [2]. Signal preprocessing improves the Signal-to-Noise Ratio (SNR) prior to feature extraction.

7.1.1 4th-Order Butterworth Band-Pass Filter

Purpose

A 4th-order Butterworth Infinite Impulse Response (IIR) band-pass filter is used to isolate sensorimotor rhythms between 8 Hz and 30 Hz.

This frequency range captures alpha and beta rhythms associated with Motor Imagery activity [15].

Mathematical Formulation

H(s)=1 / √(1 + (s/ωc)2n)

TABLE 7: BUTTERWORTH FILTER VARIABLES

| Variable | Meaning |
| --- | --- |
| H(s) | Transfer function |
| n | Filter order |
| ωc | Cutoff frequency |

For the proposed implementation:

n = 4

Justification for Butterworth Selection

The Butterworth filter was selected because it exhibits:

• Maximally flat passband response

• Minimal ripple distortion

• Smooth frequency attenuation

[2]

These characteristics preserve EEG waveform morphology while suppressing unwanted frequencies.

Implementation Logic

The filtering process consists of:

- Loading EEG signals from GDF datasets

- Extracting sampling frequency

- Applying Nyquist normalization

- Setting cutoff frequencies at 8 Hz and 30 Hz

- Computing filter coefficients

- Applying zero-phase forward-backward filtering

Zero-phase filtering prevents phase distortion in EEG signals.

7.1.2 Independent Component Analysis (ICA)

Purpose

Independent Component Analysis (ICA) performs blind source separation to isolate neural EEG activity from contaminating artifacts [16].

Raw EEG recordings contain mixed sources including:

• Electrooculography (EOG)

• Electromyography (EMG)

• Electrode motion artifacts

• Environmental interference

ICA separates these mixed signals into statistically independent components.

Mathematical Representation

X = AS

TABLE 8: ICA VARIABLES

| Variable | Meaning |
| --- | --- |
| X | Observed EEG signals |
| A | Mixing matrix |
| S | Independent sources |

ICA estimates:

W = A^-1

such that:

S = WX

Infomax ICA Algorithm

The proposed framework uses the Infomax ICA algorithm [16].

Infomax maximizes output entropy:

max H(Y)

to obtain statistically independent signal components.

Artifact Removal Procedure

- EEG signals decomposed into independent components

- Artifact-dominated components identified

- Components associated with eye blinks, eye movement, and muscle noise are removed

- Remaining components reconstructed into cleaned EEG signals

Justification for ICA

ICA was selected because it:

• Preserves neural EEG activity

• Improves classification accuracy

• Removes non-cerebral interference effectively

[16]

7.2 Feature Extraction and Feature Selection Algorithms

Feature extraction converts cleaned EEG signals into discriminative numerical representations suitable for machine learning classification.

7.2.1 Common Spatial Patterns (CSP)

Objective

Common Spatial Patterns (CSP) is a supervised spatial filtering technique designed to maximize variance differences between EEG classes [20].

The algorithm projects multi-channel EEG signals into a transformed subspace where:

• One class exhibits maximum variance

• The opposing class exhibits minimum variance

Mathematical Formulation

W = argmax (WT C1 W) / (WT C2 W)

TABLE 9: CSP VARIABLES

| Variable | Meaning |
| --- | --- |
| W | Spatial filter matrix |
| C1, C2 | Covariance matrices |

Eigenvalue Decomposition

C1 W = λ C2 W

TABLE 10: EIGENVALUE DECOMPOSITION VARIABLES

| Variable | Meaning |
| --- | --- |
| λ | Eigenvalues |
| W | Eigenvectors |

Role of Eigenvalues

• Large eigenvalues maximize variance for Class 1

• Small eigenvalues maximize variance for Class 2

The most discriminative filters are selected from both extremes.

Feature Computation

Z = WX

Feature vectors are computed using log-variance:

fi = log(var(Zi) / Σ var(Zj))

Justification for CSP

CSP was selected because it:

• Enhances class separability

• Reduces spatial redundancy

• Performs efficiently for MI classification

[20]

7.2.2 Welch’s Power Spectral Density (PSD)

Purpose

Welch’s method estimates the spectral power distribution of EEG signals [24].

The PSD captures rhythmic power variations associated with Motor Imagery tasks.

Welch Algorithm Procedure

- EEG signal divided into overlapping segments

- Each segment multiplied by a Hanning window

- FFT computed for each segment

- Individual periodograms generated

- Periodograms averaged

Mathematical Expression

PWelch(f) = (1/L) Σ Pi(f)

TABLE 11: WELCH PSD VARIABLES

| Variable | Meaning |
| --- | --- |
| L | Number of segments |
| Pi(f) | Segment periodogram |

Justification for Welch PSD

Welch’s method was selected because it:

• Reduces spectral variance

• Produces stable frequency estimates

• Improves robustness to EEG noise

[24]

7.2.3 MIBIF Feature Selection

Purpose

Mutual Information-based Best Individual Features (MIBIF) selects the most informative EEG features using information theory [26].

Mutual Information Formula

I(X;Y)=Σ p(x,y) log (p(x,y)/(p(x)p(y)))

TABLE 12: MIBIF VARIABLES

| Variable | Meaning |
| --- | --- |
| X | Feature |
| Y | Target class |

Higher mutual information indicates greater discriminative relevance.

Feature Selection Procedure

- CSP features extracted

- Mutual information computed for each feature

- Features ranked by information score

- Top-ranked features selected

Justification for MIBIF

MIBIF was selected because it:

• Reduces dimensionality

• Removes redundant features

• Improves classifier generalization

• Mitigates the curse of dimensionality

[26]

7.3 Classification and Decision Logic

7.3.1 Support Vector Machine (SVM) with RBF Kernel

Objective

Support Vector Machines classify EEG features into Motor Imagery categories by constructing an optimal separating hyperplane [8].

Linear SVM Formulation

min 1/2 ||w||²

subject to:

yi(wTxi+b) ≥ 1

TABLE 13: SVM VARIABLES

| Variable | Meaning |
| --- | --- |
| w | Hyperplane normal |
| b | Bias term |

Margin Maximization

Margin = 2 / ||w||

Larger margins improve classifier generalization.

RBF Kernel Trick

K(xi,xj)=exp(-γ||xi-xj||²)

The RBF kernel maps EEG features into a higher-dimensional space where nonlinear decision boundaries become separable.

Justification for SVM

SVM was selected because it:

• Performs effectively on small EEG datasets

• Handles nonlinear feature distributions

• Provides robust generalization

• Is highly validated in MI classification literature

[8][14]

7.3.2 Confidence Thresholding Logic

Purpose

A post-classification confidence threshold prevents unintended robotic movement.

Threshold Logic

Robotic commands are executed only if:

P(classi) ≥ 0.65

Otherwise:

Class = Rest

Functional Role

This mechanism acts as a software-level safety filter.

Only high-confidence predictions are transmitted to:

• ROS 2 control nodes

• JointTrajectoryController

• Gazebo simulation environment

This minimizes false robotic actions.

7.4 Robotic Kinematics and Inertial Modeling

7.4.1 Inverse Kinematics (IK)

Purpose

Inverse Kinematics computes the joint angles required for the UR5e end-effector to reach a target Cartesian coordinate [17].

Task Space Representation

T = [ R p ]
        [ 0 1 ]

TABLE 14: INVERSE KINEMATICS VARIABLES

| Variable | Meaning |
| --- | --- |
| R | Rotation matrix |
| p | Position vector |

IK Computation

MoveIt 2 IK solvers calculate:

θ1, θ2, θ3, θ4, θ5, θ6

for the UR5e manipulator.

Implementation Workflow

- EEG command converted into target TCP coordinate

- MoveIt 2 planning request generated

- IK solver computes joint configuration

- Joint trajectory generated

- Gazebo executes the planned trajectory

Justification for MoveIt 2 IK

MoveIt 2 provides:

• Efficient kinematic solvers

• Collision-aware planning

• Stable trajectory generation

• ROS 2 integration

making it suitable for simulation-based robotic control.

7.4.2 Parallel-Axis Theorem for Inertial Modeling

Purpose

The Parallel-Axis Theorem was used to estimate Moment of Inertia (MOI) tensors for UR5e links within the URDF model.

These inertia parameters improve the realism of Gazebo physics simulation.

Mathematical Equation

I = Icm + md²

TABLE 15: PARALLEL-AXIS THEOREM VARIABLES

| Variable | Meaning |
| --- | --- |
| I | Total moment of inertia |
| Icm | Inertia about center of mass |
| m | Link mass parameter |
| d | Distance between axes |

Role in Gazebo Simulation

Accurate inertia tensors ensure:

• Stable rigid-body dynamics

• Realistic angular acceleration

• Proper joint response

• Accurate torque propagation

within the Gazebo physics engine.

URDF Integration

Each URDF link contains:

• Inertia matrices

• Collision geometry

• Joint constraints

• Center-of-mass definitions

These parameters enable physically consistent robotic simulation.

8. Movement Mapping Logic

TABLE 16: MOVEMENT MAPPING LOGIC

| Class | Mental Command | Simulated UR5e Action |
| --- | --- | --- |
| Class 1 | Left Imagery | Base joint rotates left |
| Class 2 | Right Imagery | Base joint rotates right |
| Class 3 | Forward / Grasp | End-effector advances forward |
| Class 4 | Down / Release | End-effector lowers downward |
| Class 5 | Rest | Maintain current pose |

The command mapping layer converts classification outputs into ROS 2 trajectory commands executed through MoveIt 2.

9. Motion Planning Using MoveIt 2

MoveIt 2 is responsible for:

• Inverse kinematics

• Joint interpolation

• Collision checking

• Trajectory generation

The robotic control pipeline is represented as:

EEG → SVM → ROS2 Topic → MoveIt2 Planner → JointTrajectoryController → Gazebo

10. Software-Level Safety Mechanisms

TABLE 17: SOFTWARE-LEVEL SAFETY MECHANISMS

| Safety Mechanism | Function |
| --- | --- |
| Confidence Thresholding | Prevents uncertain movements |
| Rest-State Suppression | Cancels low-confidence actions |
| Collision Checking | Prevents unrealistic trajectories |
| Simulation-Only Execution | Eliminates physical hardware risk |

11. Expected Results and Key Performance Indicators

TABLE 18: EXPECTED RESULTS AND KPIs

| KPI | Target |
| --- | --- |
| Classification Accuracy | ≥ 75% |
| Command Success Rate | ≥ 80% |
| End-to-End Latency | ≤ 500 ms |

Latency includes:

• EEG preprocessing

• Feature extraction

• Classification

• ROS 2 transmission

• Motion initiation

[3]

12. Research Contributions

The proposed framework contributes:

- A fully simulation-based neuro-robotic architecture

- Integration of EEG classification with ROS 2 robotic control

- High-fidelity Gazebo robotic simulation

- Confidence-aware robotic decision logic

- Modular and scalable software implementation

13. Limitations and Future Scope

13.1 Current Limitations

• EEG noise sensitivity

• Subject-specific calibration

• Limited command vocabulary

• Non-invasive EEG resolution constraints

13.2 Future Improvements

Potential future extensions include:

• Deep learning-based EEG decoding [21]

• Transfer learning adaptation [6]

• Reinforcement learning-based motion planning

• Adaptive online classification

• Multi-modal biosignal fusion

14. Conclusion

This report presented a comprehensive technical methodology for implementing a Motor Imagery-based Brain–Computer Interface integrated with a simulated UR5e robotic manipulator in Gazebo. The framework combines EEG preprocessing, ICA-based artifact removal, CSP feature extraction, Welch PSD analysis, MIBIF feature selection, SVM classification, ROS 2 communication, MoveIt 2 motion planning, and Gazebo simulation into a unified neuro-robotic architecture.

The project emphasizes simulation-first validation, modular software design, confidence-aware robotic control, and realistic robotic movement emulation without requiring physical robotic deployment. The proposed methodology establishes a strong foundation for future research in neuro-robotics, intelligent robotic simulation, and brain-controlled robotic systems.

Bibliography

- Abiri, R. et al. (2019). Brain Computer Interface Paradigms.

- Bashashati, A. et al. (2007). A Survey of Signal Processing Algorithms in Brain–Computer Interfaces.

- Blankertz, B. et al. (2007). The Berlin Brain–Computer Interface.

- Wolpaw, J. et al. (2002). Brain-Computer Interfaces for Communication and Control.

- Lotte, F. et al. (2013). Review of Classification Algorithms for EEG-based BCIs.

- Côté-Allard, U. et al. (2019). Deep Learning Approaches for Transfer Learning in EEG-based BCIs.

- Dávila-Chacón, J. et al. (2018). Low-Cost Robotic Arm Control Using EEG-based BCI Systems.

- Lotte, F. et al. (2018). Review of Classification Algorithms for EEG-based BCIs.

- GitHub Repository: MOGI-ROS Simple Arm.

- GitHub Repository: Universal Robots ROS 2 Driver.

- Goldberger, A. et al. (2000). PhysioBank, PhysioToolkit, and PhysioNet.

- Nicolas-Alonso, L. et al. (2012). Brain Computer Interfaces: A Review.

- Lebedev, M. & Nicolelis, M. (2006). Brain-Machine Interfaces: Past, Present and Future.

- Lotte, F. et al. (2007). A Review of Classification Algorithms for EEG-based Brain–Computer Interfaces.

- Pfurtscheller, G. & da Silva, F. (1999). Event-Related EEG/MEG Synchronization and Desynchronization.

- Makeig, S. et al. (2002). Dynamic Brain Sources of Visual Evoked Responses.

- Huang, X. et al. (2020). 6-DOF Robotic Manipulator Simulation in Gazebo.

- Padfield, N. et al. (2019). Motor Imagery-based Brain Computer Interfaces.

- Pfurtscheller, G. & Neuper, C. (2001). Motor Imagery and Direct Brain-Computer Communication.

- Ramoser, H. et al. (2000). Optimal Spatial Filtering of Single Trial EEG During Imagined Hand Movement.

- Roy, Y. et al. (2019). Deep Learning-based EEG Analysis: A Systematic Review.

- Tangermann, M. et al. (2012). Review of the BCI Competition IV Dataset.

- RoboDK Documentation – Universal Robots UR5e Specifications.

- Welch, P. (1967). The Use of Fast Fourier Transform for the Estimation of Power Spectra.

- Ang, K. et al. (2012). Filter Bank Common Spatial Pattern Algorithm on BCI Competition IV Datasets.

- Ang, K. & Quek, C. (2006). MIBIF Feature Selection for Brain–Computer Interfaces.

- Instructables – Robotic Arm with Gripper.